Homework 1

Problem 1

Convert the following numbers from the given base to the other three bases listed in the table:

| Decimal | Binary | Octal | Hexadecimal |
| --- | --- | --- | --- |
| 144.75 | ? | ? | ? |
| ? | 10110101.101 | ? | ? |
| ? | ? | 451.3 | ? |
| ? | ? | ? | F39B.C |

- 144.75

144 / 2	= 72 	rem 0

72 / 2 	= 36 	rem 0

36 / 2	= 18 	rem 0

18 / 2	= 9 	rem 0

9 / 2	= 4 	rem 1

4 / 2	= 2	rem 0

2 / 2	= 1	rem 0

1 / 2	= 0	rem 1

0.75 * 2	= 1.5

0.5 * 2		= 1.0

Binary:		1001 0000.11

Hexadecimal	90.C

Octal:		220.6

- 10110101.101

1+4+16+32+128 + ½ + 1/8 = 181.625

Hexadecimal	B5.A

Octal:		265.5

- 451.38 = 100 101 001.011

Hexadecimal	129.6

Decimal:		1 + 8 + 32 + 256 + ¼ + 1/8 = 297.375

- F39B.C16 = 1111 0011 1001 1011.1100

Octal	171633.6

Decimal:  1 + 2 + 8 + 16 + 128 + 256 + 512 + 4096 + 8192 + 16384 + 32768 + ½ + ¼ = 62363.75

Problem 2

Perform the following binary operations:

| a) 0101 + 0011 | b) 0101 + 1011 | c) 100111 + 011011 |
| --- | --- | --- |
| d) 0101 - 0011 | e) 1011 - 0101 | f) 100111 - 011011 |
| g) 0101 * 0011 | h) 1011 * 0101 | i) 100111 * 011011 |

- 111

0101 +

0011

------

1000

- 111

0101 +

1011

------

10000

- 11111

100111 +

011011

------

1000010

- 1

0101 -

0011

------

0010

- 1

1011 -

0101

------

0110

- 11

100111 -

011011

------

001100

0101 *

0011

------

0101

0101

--------

01111

1011 *

0101

------

1011

0000

1011

----------

110111

100111 *

011011

------

100111

100111

000000

100111

100111

------------

10000011101

Problem 3

Represent the decimal numbers 694 and 835 in BCD, and show the steps necessary to find their sum.

694 -> 0110 1001 0100

835 -> 1000 0011 0101

----------------

1 1100 1001

0110

----------------

1 1111 0010 1001

0110

----------------

0001 0101 0010 1001

1    5    2    9

Problem 4

List the 6-bit binary numbers equivalent to the decimal numbers 27 through 35. Also add a parity bit in the rightmost position giving even parity to the overall 7-bit binary string.

| Decimal Number | Parity | Binary Number |
| --- | --- | --- |
| 26 | 1 | 011010 |
| 27 | 0 | 011011 |
| 28 | 1 | 011100 |
| 29 | 0 | 011101 |
| 30 | 0 | 011110 |
| 31 | 1 | 011111 |
| 32 | 1 | 100000 |
| 33 | 0 | 100001 |
| 34 | 0 | 100010 |
| 35 | 1 | 100011 |

Problem 5

What are the ASCII codes (in binary, hexadecimal, and decimal) of the following characters?

- ‘ ’ (space)	010 0000, 20, 32

- ‘+’ 		010 1011, 2B, 43

- ‘0’		011 0000, 30, 48

- ‘1’		011 0001, 31, 49

- ‘C’		100 0011, 43, 67

- ‘D’		100 0100, 44, 68

- ‘c’		110 0011, 63, 99

- ‘d’		110 0100, 64, 100

Problem 6

Demonstrate by means of truth tables the validity of the following identity:

X + YZ = (X + Y)(X+ Z)

| X | Y | Z | X + YZ | (X + Y)(X + Z) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Problem 7

Prove the identity of each of the following Boolean equations, using algebraic manipulation.

- Y + X’Z + XY’ = X + Y + Z

- ABC’ + BC’D’ + BC + C’D = B + C’D

- Y + X’Z + XY’

= (Y + XY’) + X’Z			Commutative

= (Y + X) + X’Z			Simplification

= Y + (X + X’Z)

= Y + (X + Z)				Simplification

= X + Y + Z				Commutative

- ABC’ + BC’D’ + BC + C’D

= ABC’ + BC’D’ + (BC) + C’D

= ABC’ + BC’D’ + (ABC + BC) + C’D			Absorption

= ABC’ + ABC + BC’D’ + BC + C’D			Commutative

= AB (C’ + C) + BC’D’ + BC + C’D				Distributive

= AB (1) + BC’D’ + BC + (C’D)

= AB + BC’D’ + BC + (BC’D + C’D)			Absorption

= AB + BC’D’ + BC’D + BC + C’D				Commutative

= AB + BC’ (D’ + D) + BC + C’D				Distributive

= AB + BC’ (1) + BC + C’D

= AB + BC’ + BC + C’D

= AB + B (C’ + C) + C’D					Distributive

= AB + B (1) + C’D

= (AB + B) + C’D

= B + C’D							Absorption