Homework 2

Submit handwritten Solutions

Problem 1

Find the complement of the following function:

X + Y(Z + (X + Z)’)

Problem 2

For the Boolean Function F, as given in the following truth table:

| X | Y | Z | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

- List its minterms

- List its maxterms

- List the minterms of F’

- Express F in sum-of-minterms algebraic form

- Express F in product-of-maxterms algebraic form

Problem 3

Draw the logic diagram for the following Boolean expression. The diagram should correspond exactly to the equation. Assume the complements of the inputs are not available.

B(A’C’ + AC) + D’(A + B’C)

Problem 4

Optimize the following Boolean functions by means of Karnaugh map:

- F(X, Y, Z) = Σm(0, 2, 3, 4, 6)

- F(W, X, Y, Z) = Σm(0, 2, 5, 6, 8, 10, 13, 14, 15)

- F(W, X, Y, Z) = Σm(2, 4, 9, 12, 15), with the don’t-care conditions d(W, X, Y, Z) = Σm(3, 5, 6, 13)

Problem 5

Implement the following Boolean function using NAND gates only: F(X, Y, Z) = X’Z + Y

Problem 6

Construct an exclusive-OR gate by interconnecting two three-state buffers and two inverters.

Problem 7

Obtain the truth table of the following functions, and express each function in sum-of-minterms and product-of-maxterms form:

- (AB + C)(B + AC)

- ABC’ + ABD’ + ABD + CD’

Problem 8

- Convert the following Boolean expression into SOP then into SOM: X' +X(X+Y')(Y+Z')

- Convert the following Boolean expression into POS then into POM: (AB+C)(B+AC')