Instructions:

- This exam booklet has 6 pages and 6 problems.

- The exam is closed book and notes.

- Calculators are not permitted.

- You must show your work.

- No one is allowed to leave the exam room before 5:40 pm.

Student Name:	____________________________________

(الاسم باللغة العربية)

Registration No:	____________________________________

Instructor Name:	____________________________________

Section No or Time:	____________________________________

| Problem | Max Points | Score |
| --- | --- | --- |
| 1 | 6 |  |
| 2 | 3 |  |
| 3 | 4 |  |
| 4 | 6 |  |
| 5 | 6 |  |
| 6 | 5 |  |
| Total | 30 |  |

Good Luck!

Problem 1 (6 points)

Perform the following conversions. Show the details of your solution in the space below.

- Decimal 153 to octal is _________231____________

153 / 2 = 76	rem	1

76  / 2 = 38		0

38  / 2 = 19		0

19  / 2 = 9		1

9   / 2 = 4		1

4   / 2 = 2		0

2   / 2 = 1		0

1   / 2 = 0		1

153 in binary is 010 011 001

In octal          2   3   1

- Decimal 0.6875 to binary is ______________0.1011___________________

0.6875 * 2 = 	1.375

0.357  * 2 =	0.75

0.75   * 2 = 	1.5

0.5    * 2 =	1.0

- Octal 0.513 to Hexadecimal is ___________0.A58______________

Converting from octal to binary 			0.101 001 011

Collect four digits and add padding 0’s	0.1010 0101 1000

Convert from binary to hexadecimal		0.A    5    8

- Hexadecimal 306.D to binary is ____0011 0000 0110.1101______________

Convert every hexadecimal digit to four binary digits

3    0    6   .D

0011 0000 0110.1101

Problem 2 (3 points)

Without converting to binary or decimal, add the two packed BCD numbers 0011 1001 and 0010 0001. Show the details of your solution.

11    1

0011 1001	this is 39

0010 0001	this is 21

-----------

1 1010  larger than nine, so we add 0110

0110

-------------

0110 0000 this is 60

Problem 3 (4 points)

Using NAND gates only, implement the function F = A(CD + B’)

Problem 4 (6 points)

| Consider the Boolean function: F = X Y + X' Z X | Y | Z | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

- Write the truth table for F in the table shown to the right.

- Find the complement of F in simplified POS form.

F’ = (XY + X’Z)’

F’ = (X’ + Y’) · (X + Z’)

- Express F as a sum of minterms.

F = X’Y’Z + X’YZ + XYZ’ + XYZ

- Implement the Boolean function (F = X Y + X' Z) using basic logic gates.

Problem 5 (6 points)

- For the following sum of minterms expression, derive a simplified sum of products (SOP) expression using a Karnaugh map. Furthermore, in the table on the right, identify ALL the prime implicants, indicating which are essential.

F (A,B,C,D)= ∑m (2,5,7,8,9,10,12,13,14,15)

|  | prime implicants | Essential? |  |
| --- | --- | --- | --- |
|  |  | yes | no |
|  | B’CD’ |  |  |
|  | BD |  |  |
|  | AC’ |  |  |
|  | AB |  |  |
|  | AD’ |  |  |

Simplified SOP Expression:  B’CD’+BD+AC’+AB  or  B’CD’+BD+AC’+AD’

- Obtain a simplified SOP expression for the following K-map:

Simplified SOP Expression: _____B’D’+AD’+BCD_____________________

Problem 6 (5 points)

Design a combinational circuit that outputs 1 if its 4-bit input number is divisible by (4)10 and outputs 0 otherwise.

Truth Table:							Optimization (K-map):

| A | B | C | D | F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |  |  |  |
| 0 | 0 | 0 | 1 | 0 |  |  |  |
| 0 | 0 | 1 | 0 | 0 |  |  |  |
| 0 | 0 | 1 | 1 | 0 |  |  |  |
| 0 | 1 | 0 | 0 | 1 |  |  |  |
| 0 | 1 | 0 | 1 | 0 |  |  |  |
| 0 | 1 | 1 | 0 | 0 |  |  |  |
| 0 | 1 | 1 | 1 | 0 |  |  |  |
| 1 | 0 | 0 | 0 | 1 |  |  |  |
| 1 | 0 | 0 | 1 | 0 |  |  |  |
| 1 | 0 | 1 | 0 | 0 |  |  |  |
| 1 | 0 | 1 | 1 | 0 |  |  |  |
| 1 | 1 | 0 | 0 | 1 |  |  |  |
| 1 | 1 | 0 | 1 | 0 |  |  |  |
| 1 | 1 | 1 | 0 | 0 |  |  |  |
| 1 | 1 | 1 | 1 | 0 |  |  |  |

Logic Diagram Using NOR Gates Only: