Homework #2

Computer Organization

Q1. What are the maximum and minimum integers for 32-bit signed numbers?

Q2. For the following functions,

| Function | Bnegate | Operation |
| --- | --- | --- |
| and | 0 | 00 |
| or | 0 | 01 |
| add | 0 | 10 |
| sub | 1 | 10 |
| slt | 1 | 11 |

Design the MIPS 1-bit ALU slice for the least significant bits.

Q3. Design optimized iterative multiplication hardware for multiplying two 32-bit integers.

Q4. Now for division, design the division hardware for dividing two 32-bit integers.

Question 5:

Represent the following in single-precision IEEE floating point. Give your answers in hexadecimal:

a) 2007.0

b) -4014.0

c) -0.00390625 	(Hint: -2-8)

d) 16777215.0 	(Hint: 224 -1)

e) 16777216.0 	(Hint: 224)

Question 6:

Convert the following single-precision floating point numbers (given in hexadecimal) to decimal:

- 0x3e800000

- 0xbec00000

- 0x46800000

- 0xc67ffc00

- 0x46800200

Question 7:

Given the following two single-precision IEEE floating-point numbers:

x = 0x3c800000 and y = 0x48800000

Compute the following showing all work: (ignore guard, round and sticky bits)

a) x + y

b) x × y

c) x – y

Question 8:

Calculate the relative speeds of a 32-bit adder using ripple carry only, ripple carry of eight 4-bit groups that use carry lookahead, ripple carry of two 16-bit groups that use carry lookahead, and the three-level carry-lookahead scheme. Assume that every gate delay is T seconds.

Question 8:

Design a 4-bit barrel shifter that has the following control signals:

| sa0, sa1 | Shift amount |
| --- | --- |
| A | 0 perform logical shift 1 perform arithmetic shift |
| L | 0 right shift 1 left shift |

Question 9:

Given the following two implementations P1 and P2 of the same ISA,

| Class | CPI on P1 |  |  | CPI on P2 | Instr. Count |  |
| --- | --- | --- | --- | --- | --- | --- |
| A | 1 |  |  | 2 | 1000 |  |
| B | 2 |  |  | 2 | 2000 |  |
| C | 3 |  |  | 2 | 1000 |  |
| D | 4 |  |  | 4 | 3000 |  |
| E | 3 |  |  | 4 | 2000 |  |
|  |  | P1 Clock Rate | P2 Clock Rate |  |  |  |
|  |  | 4 GHz | 6 GHz |  |  |  |

- What is the average CPI for P1?

- What is the average CPI for P2?

- What is the CPU execution time for a program that has the instruction counts shown in the above table on P1?

- What is the CPU execution time for a program that has the instruction counts shown in the above table on P2?

- Which processor is faster and by what percentage?