#### 0907335 Computer Organization (Spring 2008)

#### Quiz 1

الاسم:					رقم التسجيل:	 		رقم الشعبة:	4

===========================================================================

Instructions: Time 10 minutes. Closed books and notes. No calculators. No questions are allowed.

Q. Assume that the contents of some registers and memory locations (in hexadecimal numbers) are initially as shown in the following table:

| Register or Memory | Contents |
| --- | --- |
| $s0 | 0x00001ac0 |
| $s1 | 0x11111111 |
| $s2 | 0x12345678 |
| Mem[0x00001ac0] | 0x87654321 |

Complete the table below after the following MIPS instruction sequence is executed.

lw	$t0, 0($s0)

add	$s3, $s1, $s2

ori	$s4, $s2, 1

bne	$t0, $t0, L1

sw	$s0, 4($s0)

L1:

| Register or Memory | Contents |
| --- | --- |
| $t0 | 0x87654321 |
| $s3 | 0x23456789 |
| $s4 | 0x12345679 |
| Mem[0x00001ac4] | 0x00001ac0 |

Q. Write a procedure, Cap, in MIPS assembly language. The procedure should take a single argument that is a pointer to a null-terminated string in register $a0. The Cap procedure should convert this string to capital letters. For example, if the argument to Cap points to the string “School”, then Cap will change this string to “SCHOOL”. Hint: oring a letter with 0x20 converts to it to the capital case.

Cap:

lbu $t0, 0($a0) 			# load one character

beq $t0, $zero, c_end 	# end if null

andi $t0, $t0, 0xdf		# convert it to capital

sb $t0, 0($a0) 			# store it back

addi $a0, $a0, 1 			# point to next char

j Cap					# repeat

c_end:

j $ra