#### 0907432 Computer Design (Fall 2007)

#### Quiz 1C

الاسم:					رقم التسجيل:	 		رقم الشعبة:

===========================================================================

Instructions: Time 12 minutes. Closed books and notes. No calculators. Please answer all problems in the space provided. No questions are allowed.

<Good Luck>

Q. How do you take advantage of parallelism on the system level?

____Replicate components such as processors, disks, and memory modules. _

Q. Given the following specifications,

|  | Processor | Memory | Disk |
| --- | --- | --- | --- |
| Type | Pentium 4 | DDR-SDRAM | 7200 rpm |
| Count | 3 | 8 | 3 |
| FIT (per unit) | 5000 | 5000 | 15000 |

what is the system availability if any fault takes one day to repair?

System FIT = 3 * 5000  +  8 * 5000  +  3 * 15000 = 100,000

MTTF = 1000,000,000 / 100,000 = 10,000 hours

Availability = 10,000 / ( 10,000 + 24 )

Q. What is the main advantage of using a 3-operand ISA over 2-operand ISA?

____Gives more flexibility to the programmer/comiler.__________________

Q. Using pipeline diagrams, find how many cycles are needed to execute the following code sequence. Assume that split cycle register file is used, stalls are used to solve data hazards, branch instructions are resolved in the Execute stage, branch target address is found in the Decode stage, and predict taken scheme is used.

1 0 0

1 2 3 4 5 6 7 8 9 0 1 2

lw 	r1, 0(r2)       F D E M W

sw	r1, 0(r3)         F D - - E M W

bne	r1, r1, skip        F - - D E M W

addi	r3, r3, 1                 F n F D E M W         n: converted to null

skip:

Target instruction                F n n n n

12CYLCES