Computer Design

CPE 432

Homework 1

- Solve the following problems from Chapter 1 of your textbook: 1.1, 1.4, 1.9, 1.10, and 1.14.

- Show how to implement the addressing modes in Figure B.6 using the first three addressing modes.

- Write code sequences to implement D=(A-B)*(A-C); on the four ISA classes.

- Using pipeline diagrams, find how many cycles are needed to execute the following code sequence.

  - When stalls are used to solve hazards and branch instructions are resolved in the Execute stage.

  - When full forwarding paths are used plus stalls (when needed), one branch delay slot, and branch instructions are resolved in the Decode stage.

lw 	r1, 0(r2)

add	r3, r1, r1

sw	r3, 0(r2)

beq	r4, r4, skip

andi	r3, r3, 0

skip:	sw	r3, 4(r2)