University of Jordan

Computer Engineering Department

CPE439

Computer Design Lab

Experiment 2

Three-Port Register File

It is required to construct and test a Verilog module for the three-port register file described in Section 5.2 of your Computer Organization and Design textbook. The symbol for this register file is shown below.

This register file has thirty two 32-bit registers. Its delay from register select to output is 5 units.

Hint: You can construct this register file using behavioral modeling similar to the code below.

//

// The following test module illustrates how to use the register file below

//

module test;

reg clock, wt;

reg [7:0] addr;

reg [15:0] Din;

wire [15:0] Dout;

RF rf1(clock, wt, addr, Din, Dout);

initial #100 $finish;

initial begin: Init

$display ("time clock wt addr Din Dout");

$monitor ("%0d %b %b %h %h %h", $time, clock, wt, addr, Din, Dout);

clock = 1'b0;

wt = 1'b0;

addr = 8'h00;

Din = 16'h1234;

#10 clock = 1'b1;

#10 clock = 1'b0;

#10 wt = 1;

#10 clock = 1'b1;

#10 clock = 1'b0;

#10 addr = 100;

end

endmodule

//

// 16-bit Register File, size = 2 bytes * 256 locations

//

module RF(clock, wt, addr, Din, Dout);

input clock, wt;

input [7:0] addr;

input [15:0] Din;

output [15:0] Dout;

reg [15:0] Dout;

reg [15:0] MA [255:0];

initial begin: InitContents

// initialize the contents here, if needed

MA[100] = 16'h0003;

MA[101] = 16'h0011;

end

always @(addr) #4 assign Dout = MA[addr];

always @(posedge clock)

if (wt == 1)

#1 MA[addr] = Din;

endmodule