University of Jordan

Computer Engineering Department

CPE439

Computer Design Lab

Experiment 1

32-Bit ALU

It is required to construct and test a Verilog module for the 32-bit ALU described in Section 4.5 of your Computer Organization and Design textbook. The symbol for this ALU is shown below.

The ALU should support the following five operations:

| ALU Operation | Function | Delay |
| --- | --- | --- |
| 000 | And | 5 |
| 001 | Or | 5 |
| 010 | Add | 12 |
| 110 | Subtract | 12 |
| 111 | Set on less than | 12 |

Hint: You can construct this ALU using behavioral modeling similar to the code below.

//

// The following test module illustrates how to use the ALU below.

//

module test;

wire [15:0] f;

wire cout;

reg [15:0] a, b;

reg cin;

reg [3:0] s;

ALU alu1(f, cout, a, b, cin, s);

initial #100 $finish;

initial begin: Init

$display ("time a b cin s f cout");

$monitor ("%0d %h %h %b %b %h %b", $time, a, b, cin, s, f, cout);

a = 16'h9111;

b = 16'h6666;

cin = 1'b0;

s = 4'b0001;

end

endmodule

//

// 16-bit ALU

//

module ALU (f, cout, a, b, cin, s);

output [15:0] f;

output cout;

reg [15:0] f, t;

reg cout;

input [15:0] a, b;

input cin;

input [3:0] s;

always @(a or b or cin or s) begin

#12;

case (s)

4'b0000: {cout, f} = a + cin;         // transfer A or increment A

4'b0001: {cout, f} = a + b + cin;     // addition

4'b0010: begin                        // subtraction

t = ~b;

{cout, f} = a + t + cin;

end

4'b0011:                              // decrement or transfer A

if (cin == 1'b0)

{cout, f} = a + 16'hffff;

else

{cout, f} = a;

4'b0100: {cout, f} = a & b;           // and

4'b0101: {cout, f} = a | b;           // or

4'b0110: {cout, f} = a ^ b;           // xor

4'b0111: {cout, f} = ~a;              // not

4'b1000: {f, cout} = {1'b0, a, 1'b0} >> 1;  // shift right

4'b1100: {cout, f} = a << 1;          // shift left

default: {cout, f} = 17'bzzzzzzzzzzzzz;

endcase

end

endmodule