University of Jordan

Computer Engineering Department

CPE439

Computer Design Lab

Experiment 5

Five-Stage Pipeline Datapath – Part III

It is required to upgrade the Verilog module you have developed in Experiment 4 to solve RAW data hazards. In order to solve these hazards, the pipeline should be modified as follows:

- The decode stage should detect load-use scenario and stall the instructions in the fetch and decode stage when this scenario occurs.

- The execute stage should be modified to get forwarding paths from the memory and write-back stages.

- The memory stage should be modified to get forwarding path from the write-back stage.

- The decode stage should be modified to get forwarding paths from the memory stage. This forwarding is needed to solve the RAW hazard that occurs when an ALU instruction is followed by a branch instruction, as in the code sequence below. Note that the branch instruction in such sequence must stall for one cycle.

lw	R1, 8(R0)

loop:	addi	R1, R1, -1

bne	R1, R0, loop

sw	R1, 8(R0)

To test your design, make the following initializations:

- Program Counter = 0x00000000

- All 32 registers are initialized to the value 0x00000000

- The data memory is initialized such that each location has a number greater than the previous location by 1 and location 0x00 is initialized to 0x00000000, e.g., location 0x08 should be initialized to 0x00000002, and location 0x0c is initialized to 0x00000003.

- The instruction memory should be initialized to hold the following code sequence, starting at location 0x00:

lw	R1, 4(R0)

add	R1, R1, R1

add	R2, R0, R1

add	R3, R1, R2

sw	R3, 0(R3)

When you simulate this module using proper test code for 9 cycles, the word at memory location 0x04 should hold the value 0x00000004.