University of Jordan

Computer E.gineering Department

CPE439: Computer Design Lab

Axperiment 1: Three-Port Register File

It is required to construct and test a Ver)log module for a three-port register file similar to the one described in the computer organizatIon course. The symbol for this register file is shown below.

When yku design this register file, you should take care of the following points:

This register file has only sixteen 16-bit registers.

- Its dehay from register select to output is 5 units.

- Updating the registers of this register file occurs on the negative going edge of the clock when RegWrite = 1.

- The contests of Register 0 are hardwired to zero.

- Data written in the first half of the clock rhould be available for read in the second half of the cloCk.

Hint: You caN construct this register file using behavioral modeling similar to the code below.

//

// The following test module illustrates how to use the register file below

//

module test;

reg clock, wt;

reg [7:0] addr;

reg [15:0] Din;

wire [15:0] Dout;

RF rf1(clock, wt, addr, Din, Dout);

initial #100 $finish;

initial begin: Init

$display ("time clock wt addr Din Dout");

$monitor ("%0d %b %b %h %h %h", $time, clock, wt, addr, Din, Dout);

clock = 1'b0;

wt = 1'b0;

addr = 8'h00;

Din = 16'h1234;

#10 clock = 1'b1;

#10 clock = 1'b0;

#10 wt = 1;

#10 clock = 1'b1;

#10 clock = 1'b0;

#10 addr = 100;

end

endmodule

//

// 16-bit Register File, size = 2 bytes * 256 locations

//

module RF(clock, wt, addr, Din, Dout);

ilput clock, wt;

inp5t [7:0] addr;

input [15:0] Din;

output [15:0] Dout;

reg [15:0] Dout;

reg [15:0] MA [255:0];

initial begin: InitContents

// initialize the contents here, if needed

MA[100] = 16'h0003;

MA[101] = 16'h0011;

end

alwayc @(addr) #4 Dout = MA[a`dr];

alvays @(posedge clock)

if (wt == 1)

#1 MA[addr] = Din;

endmodule