University of Jordan

Computer Engineering Department

CPE439: Computer Design Lab

Experiment 4: Five-Stage Pipeline Datapath – Part II

It is required to upgrade the Verilog module you have developed in Experiment 3 to support additional instruction types. In order to support the additional instructions types listed below, the pipeline should be modified as follows:

- The fetch stage should be able to fetch the next sequential instruction, the branch target instruction, and jump target instruction (immediate or stored in one of the registers).

- The decode stage should resolve branches and find branch and jump target addresses.

- The execute stage should accept immediate operands.

- The write back stage should be extended to allow saving the return address.

- For jumps and taken branches, the successor instruction must be flushed.

To test your design, make the following initializations:

- Program Counter = 0x0000

- All 16 registers are initialized to the value 0x0000

- The instruction memory should be initialized to hold the following code sequence, starting at location 0x00:

L1: 	addi	R2, R0, #2

bne	R0, R3, L1

ori	R3, R0, #7

slti	R1, R2, #7

beq	R2, R0, L1

andi	R3, R3, #3

jal	L2

jr	R0

L2:	add	R5, R1, R2

When you simulate this module using proper test code for 12 cycles, R3 and R5 should hold the value 0x0003.